Token/Password for pushing:

#1: 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

#2: 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

Op-Amp Comparator Notes -> op-amp comparator explained

Voltage comparator -> finding out when (in time) an input voltage is above or below a reference voltage (Vin > Vref or Vin < Vref)

EX: Low battery warning circuit -> compare V(battery) to a battery low threshold V(low)

What is an Op-Amp?

IC’s -> integrated circuits

Op Amp -> operational amplifier

Amplifier: Circuit that increases the strength/magnitude of an electrical signal (the voltage amplitude of the signal when graphed)

Op Amp -> designed to do math (represented by a triangle symbol)

2 inputs & 1 output

2 Inputs: Inverting input (-), non-inverting input (+)

Powered by a dual-rail DC supply (connects to V+ and V-) -> provides +ve & -ve voltages

Op-amps don’t draw current from the input terminals

Gain -> amplifies the difference between the two inputs

Vinverting - Vnon-inverting is the difference we care about -> gain = Voutput/(Vinverting - Vnon-inverting)

Used in open loop  (no feedback connecting output to input) or closed loop configuration (output connects to inverting input through a resistor)

Open loop: extremely high gain (small differences will bring the input all the way to the +ve or -ve supply voltage) -> useful in voltage comparators (to detect which input is higher) -> since output = Vinverting - Vnon-inverting, if output is negative (Vnon-inverting > Vinverting) then gain will be negative and the signal will go to the negative max V-, and if the output is positive (Vinverting > Vnon-inverting), then the gain will be positive and the signal will go to the positive max V-

Closed loop: negative feedback helps control the gain

Golden rules (for closed loop):

- No current enters or leaves op-amp inputs

- Input voltages are equal with negative feedback

Op-Amp Comparator Notes (continued)

LM358 -> 2 op-amps in 1 IC -> we connect V- to ground and V+ to the positive supply

Inverting input (in-) -> feed in a fixed reference voltage (get the fixed reference voltage using resistors, splitting the voltage drop between the positive supply and ground)

Non-inverting input -> comes from sensor

Functionality: if Vin < V ref, Vout = 0. If Vin > V ref, Vout = +Vcc -> doesn’t jump all the way to the top (depending on which specific op-amp you’re using)

Questions:

What should Vcc be?

How do we know what “high” is? -> scale: 500 mV to 4500 mV? -> use V=IR

Can we take the output signal and put it through an and gate (to also take into account the current sensor)?

How do we represent opening the BSPD fault (and stopping the battery) on a circuit diagram?

How to pick a good op-amp?

How do we add a 1 second timer?